JCB 3CX Backhoe Loader

Hydraulic Cylinders & Technical Specifications

# 1. Types of Hydraulic Cylinders

The JCB 3CX uses double-acting hydraulic cylinders throughout the machine. This means pressurised oil can be applied to both sides of the piston, allowing both extension and retraction under power — giving full control in both directions. There are 6 main hydraulic cylinder positions on the 3CX:

| Cylinder | Qty | Location | Function |
| --- | --- | --- | --- |
| Boom Cylinder | x1 | Rear boom arm | Raises / lowers the boom |
| Dipper / Stick Cylinder | x1 | Dipper arm | Extends / retracts the dipper |
| Bucket Cylinder | x1 | Rear bucket | Curls / dumps the excavator bucket |
| Loader Lift Cylinders | x2 | Front loader arms | Raises / lowers the loader |
| Loader Bucket Cylinder | x1 | Front bucket linkage | Tilts the loader bucket |
| Stabiliser Cylinders | x2 | Outriggers | Deploys the stabiliser legs |

# 2. Hydraulic System Specifications

JCB cylinders feature hose protection and structural steel tubing. The hydraulic system runs on triple gear pumps via an open-centre circuit:

| Parameter | Value |
| --- | --- |
| System Type | Open Centre |
| Pump Type | Triple Gear Pumps (x3) |
| Total Pump Output | 39 gpm (148 lpm) |
| System Relief Pressure | 3,650 psi (252 bar) |
| Filter Rating | 5/12 micron |
| Hydraulic Tank Capacity | 18 gal (68 litres) |
| Total Hydraulic System Capacity | 31 gal (117 litres) |

# 3. Engine Cylinder Measurements

## 3.1 JCB 3CX-14 — EcoMAX Engine

The standard 3CX-14 is fitted with the JCB EcoMAX turbocharged diesel engine:

| Specification | Value |
| --- | --- |
| Engine Model | JCB EcoMAX |
| Cylinders | 4 |
| Displacement | 4.4 litres |
| Bore | 4.05 in (102.9 mm) |
| Stroke | 5.0 in (127 mm) |
| Rated RPM | 2,200 RPM |
| Gross Power | 74 hp (55 kW) |
| Net Power | 71 hp (53 kW) |
| Peak Torque | 295 lbf·ft (400 Nm) @ 1,200 RPM |

## 3.2 JCB 3CX Compact — Kohler Engine

The compact variant uses a smaller Kohler-based diesel engine:

| Specification | Value |
| --- | --- |
| Engine Model | JCB Diesel by Kohler |
| Cylinders | 4 |
| Displacement | 2.4 litres |
| Bore | 3.4 in (88 mm) |
| Stroke | 4.0 in (102 mm) |
| Rated RPM | 2,350 RPM |
| Gross Power | 74 hp (55 kW) |
| Peak Torque | 221.3 lbf·ft (300 Nm) |

# 4. Key Machine Dimensions (3CX-14)

| Dimension | Imperial | Metric |
| --- | --- | --- |
| Transport Length | 23 ft 7 in | 7.19 m |
| Transport Height (Standard Backhoe) | 11 ft 3 in | 3.43 m |
| Transport Height (Extradig) | 11 ft 7 in | 3.53 m |
| Height to Top of Cab | 9 ft 3 in | 2.81 m |
| Overall Width with Bucket | 7 ft 8 in | 2.45 m |
| Wheelbase | 7 ft 1 in | 2.17 m |
| Max Dig Depth (14 ft) | 14 ft 0 in | 4.27 m |
| Max Horizontal Reach | 17 ft 7 in | 5.37 m |
| Max Discharge Height | 8 ft 11 in | 2.74 m |

# 5. Special Cylinder — SRS Vibration Compensator

The JCB 3CX also features an SRS (Smooth Ride System / rear linkage vibration compensator) that prevents cargo spilling from the loader bucket during travel. This system consists of:

- 1 x dry nitrogen gas cylinder

- 1 x hydraulic accumulator

Together these absorb boom oscillation when the machine is travelling at speed with a loaded front bucket.

Note: Exact bore and stroke measurements for individual hydraulic work cylinders (boom, dipper, bucket) vary by variant and are not published in JCB's public specification sheets. For precise figures, consult a JCB dealer or the official JCB service manual for your specific machine serial number.